# Briefing Document: Bus Stop Bypasses, Boarders and Islands – Accessibility and Safety Concerns

## Introduction

This document summarises key information regarding bus stop designs that incorporate cycle lanes, specifically focusing on the safety and accessibility challenges they pose for blind and partially sighted people and other potentially vulnerable users. It sets out the approach of Thomas Pocklington Trust to these issues and mitigations that should be considered as part of any designs.

Blind and partially sighted people have the right to access bus services from a safe, accessible space that meets their specific needs.

The designs in question are:

- A Bus Stop Bypass (or floating bus stop) is where a cycle lane is cut into the pavement which runs behind a bus stop or bus shelter. The cycle lane runs parallel with the road and turns the bus stop into an ‘island’ that is separate from the rest of the pavement. In order for bus passengers to access the bus stop they would need to cross the live cycle lane.

- A Bus Stop Boarder also sees a cycle lane running between the pavement and the place where the bus stops, but the difference is that there is no dedicated bus stop island where pedestrians wait for the bus. Bus passengers attempting to access the bus or alight from it must do so from the cycle lane which becomes a shared use area at this point. Therefore, when getting on or off the bus they are effectively stepping directly from, or onto, a live cycle lane.

- A Bus Island is similar to a bus bypass, but in this case the bus shelter is located towards the back of the footway. The cycle lane runs between the pavement and an island which bus passengers use to get on and off the bus. As with a bus stop bypass, the passenger needs to cross the live cycle lane to traverse from the bus shelter to the island in order to board the bus.

## Key themes

- Accessibility concerns for blind and partially sighted users: There is an inherent danger and inaccessibility of bus stop bypasses, boarders, and islands for blind and partially sighted individuals. These designs force vulnerable pedestrians to navigate active cycle lanes, creating conflict and safety risks.

- Conflicts between active travel and inclusivity: The push for active travel, primarily cycling, often seems to come at the expense of accessibility for vulnerable road users. There can be a tension between the aims of encouraging cycling and ensuring safety and inclusivity for all, in particular disabled pedestrians.

- Inadequacy of standard designs: Existing design guidelines (such as LTN 1/20) do not adequately address the needs of all users. Specifically, the recommended minimum widths for bus islands are found to be insufficient, especially for wheelchair users, assistance dogs, and those with mobility aids.

- Importance of inclusive design principles: Design decisions must prioritise safety for pedestrians and ensure all user needs are met.

- Lack of standardisation: There is not a standard layout for Floating Island Bus Stops, so users "have to establish where to wait and board the bus." This can lead to safety issues for all users, especially those with visual impairments.

## Headline recommendations

It is clear that a fundamental shift is needed in the planning and design of infrastructure that incorporates cycle lanes, and the needs of those with visual impairments and other mobility issues must be prioritised.

The current designs of bus stop bypasses, boarders, and islands are not fit for purpose. Our key recommendations are:

- Prioritise pedestrian safety: The design of bus stops must not compromise the safety of pedestrians, especially blind and partially sighted individuals and other vulnerable users. Shared use spaces that introduce a risk to users should be avoided.

- Re-evaluate current guidelines: Existing design guidelines need to be reviewed and revised to include accessibility as a primary consideration. Minimum widths, tactile surfaces, and clear delineation between spaces for different users should be based on real-world evidence and the needs of all users.

- Consult user groups: Input from blind and partially sighted users, disability advocacy groups, and other relevant stakeholders should be incorporated into all stages of planning and design.

- Invest in further research: More research is urgently needed on methods to reduce conflicts between cyclists and pedestrians, particularly at bus stops, including how to reduce the speed differential between them, and better ways to help people identify which bus has arrived at the stop. Further research into the dimensions of the platform area of bus stops is also needed.

- Avoid Bus Stop Bypasses and Boarders: Where feasible, avoid using  bus stop bypasses (floating bus stops) and bus boarders completely, instead focusing on designs that minimise conflict and offer clear, safe routes for both pedestrians and cyclists.

- Standardisation: A standard design for bus stops that incorporate cycle lanes needs to be created and should be deployed across local transport authority footprints for consistency.

## Potential mitigation strategies

While, for the reasons set out in this briefing, we remain concerned about these bus stop designs we recognise the potential for mitigations such as consistent tactile paving, clear delineation between cycle paths and pedestrian zones and reduced cyclist speeds.

While these mitigations may assist, they do not remove the inherent risks presented by the designs and the priority should always be avoiding conflict in design. We would encourage planners to work through a hierarchy of considerations:

- Involve disabled people: Blind and partially sighted people should be involved in the design and planning process to test mitigations.

- Always prioritise pedestrian safety: Locations must be designed to prioritise the safety of pedestrians, particularly vulnerable groups.

- Avoid shared space: Mixing cyclists and pedestrians is not a good solution and should be avoided.

- Alternative routes: Consider creating parallel routes for cyclists, to avoid conflict with pedestrians.

- Single direction lanes: If cycle lanes must come into conflict with pedestrians, these should be single direction lanes to minimise the risks.

- Consistent design: Consistent and standardised designs for bus stops and crossings are necessary for safety.

- Tactile paving: Tactile paving should be installed consistently at all crossings, at the edges of cycle lanes, and at the start and end of segregated cycle-footways.

- Clear delineation: There must be clear delineation between cycle lanes and footways. A raised delineator rather than a painted line is needed.

- Reduce cyclist speed: Measures should be implemented to control cyclist speed near bus stops and crossings including colour, speed reduction measures, signage, reducing cycle lane widths and ramps. Rumble strips near to crossing points to reduce cyclist speed should be considered.

- Minimise opportunities for bad behaviours: Where possible designs should prevent unintended consequences or encouraging bad behaviours such as short cuts. The use of chicanes and barriers could be considered to ensure cyclists and pedestrians use interaction points as intended.

- Utilise crossings to access islands: If bus islands are used, ensure that appropriate pedestrian crossings are included for access including tabletop zebra crossings as a minimum and ideally pedestrian controlled signals. These could utilise new technology to detect cyclists and improve safety for all.

- Increase visibility: Improve visibility of the bus stop location, crossings and tactile paving.

- Bus stop location: Ensure bus stops are located so they are easy to identify, and that bus stop flags are visible from the footway.

- Bus stops should be fully accessible: There should be adequate space for all passengers, including seating, and buses should always stop in the same place.

- Improved signage: Clear and comprehensive signage and labeling should guide users, including audio visual announcements.

## Useful documents

- Department for Transport LTN 1/20 Cycle Infrastructure Design

- Department for Transport,  Inclusive Transport Strategy

- Department for Transport: Inclusive Mobility: A Guide to Best Practice on Access to Pedestrian and Transport Infrastructure

- RNIB: Key Principles of Inclusive Street Design (RNIB)

- Guide Dogs and UCL: Designing for Inclusion

- Transport for London Bus stop bypass safety review 2024

## Annex

The annex below includes examples of mitigations or suggestions made at specific sites around the United Kingdom. Thomas Pocklington Trust does not endorse these documents and shares them as background information to inform local thinking.